MISH 19

Equipment Maintenance & Inspection

| DOCUMENT | REVISION | EFFECTIVE DATE | APPROVED BY |
| --- | --- | --- | --- |
| MISH 19 | 2 | 04/07/2026 | Jeremy Thamert |

﻿MH CONSTRUCTION INDUSTRIAL SAFETY AND HEALTH PROGRAM

Title

EQUIPMENT MAINTENANCE AND INSPECTION

Number

MISH 19

Revision 1 Approved By Jeremy Thamert                                              03/01/2025 Name                                                      Date Effective Date 3/1/2025 Page 1 of 9

1.0  PURPOSE

To establish the system to inspect and maintain vehicles and heavy construction equipment.

2.0  RESPONSIBILITY

2.1  The Key Supervisor shall be responsible for the following:

2.1.1  Maintaining vehicles and equipment in accordance with applicable Occupational Safety and Health Administration/Washington Industrial Safety and health Act (OSHA/WISHA) and American national Standards Institute (ANSI) standards.

2.1.2  Conducting inspections of equipment, and correcting deficiencies noted during the inspections.

2.2  Employees shall be responsible for conducting daily inspections of equipment they are operating, and for reporting deficiencies to their supervisor.

3.0  DEFINITIONS

Heavy Construction Equipment:  Mobile cranes, crawler cranes, tractors, compactors, fork lifts, graders, scrapers, off-highway trucks, etc.

Motor Vehicles:  Any vehicles that operate on a construction site.

Preventive Maintenance:  Practice of utilizing periodic inspections, adjustments, lubrication, and minor repair of equipment for the purposes of extending its useful life and reducing the frequency and magnitude of repairs.

Scheduled Maintenance:  Regularly scheduled inspections, tests, lubrications, adjustments, cleaning, and tightening of various components of a piece of equipment.

4.0  GENERAL REQUIREMENTS

4.1  Equipment maintenance and inspection shall comply with the following statutory requirements:

•  29 Code of Federal Regulations (CFR) 1926, Subpart O; Motor Vehicles, Mechanized Equipment and Marine Operations

•  29 Code of Federal Regulations (CFR) 1910, Subpart N; Materials Handling and Storage

•  29 Code of Federal Regulations (CFR) 1910, Subpart F; Powered Platforms, Manlifts, and Vehicle-Mounted Work Platforms

•  Washington Administrative Code (WAS) 296-155, Part M; Motor Vehicles, Mechanized Equipment and Marine Operations.

4.2  On-site preventive maintenance of equipment and vehicles shall be performed by qualified personnel.

4.3  Motor vehicles and heavy equipment shall be maintained and inspected monthly in conformance to the manufacturer’s recommendations, and documented on equipment-specific safety check and/or inspection report forms (see attachments 19-A).

4.4  Prior to heavy equipment being brought onto a project, an inspection of the equipment shall be performed and complete the applicable safety check and inspection report forms.  The forms shall be submitted to maintenance department.  Deficiencies noted during the inspection shall be corrected prior to the equipment being brought onto the project.

4.5  MH Construction shall notify the owner or general contractor when heavy equipment is to be brought onto a project.

4.6  The key supervisor shall maintain manufacturer’s recommendations and inspection checklists onsite for review.

5.0  PROCEDURE

5.1  MOTOR VEHICLES

5.1.1  Vehicles being operated shall be checked at the beginning of each shift to ensure that the following parts, equipment, and accessories are in safe operating condition and free of apparent damage that could cause failure while in use.

•  Brakes (including trailer brake connections, parking brake, and emergency stopping systems.

•  Tires

•  Horn

•  Steering mechanism

•  Coupling devices

•  Seat belts

•  Operating controls

•  Safety devices

5.1.2  Lights, reflectors, windshield wipers, defrosters, fire extinguishers, steps, and hand-holds for vehicle access shall also be inspected where such equipment is necessary.

5.1.3  Defects noted during the motor vehicle inspection shall be corrected before the vehicle is placed into service.

5.2  HEAVY CONSTRUCTION EQUIPMENT:  The following are specific requirements relating to heavy equipment.

5.2.1  Crawler, locomotive, and truck cranes in use shall meet the applicable requirements for design, inspection, construction and testing, maintenance and operation as prescribed in the ANSI B30.5.

NOTE:	Daily inspections shall be conducted on all critical items such as brakes, crane hooks, ropes, hydraulic and pneumatic cylinders, and hydraulic and pneumatic relief pressure valves.  These inspections shall be documented and the records kept available for review.

5.2.2  Lift Trucks:  Operation of powered industrial trucks may be hazardous if maintenance is neglected or repairs, rebuilds, or adjustments are not performed in accordance with the manufacturer’s design criteria.

5.2.2.1  A scheduled, planned maintenance, lubrication, and inspection system shall be followed on all powered industrial trucks.

5.2.2.2  In an industrial truck is found to be in need or repair or is in any way unsafe or contributes to an unsafe condition, the truck shall not be operated until it has been restored to safe operating condition.

5.2.2.3  Only qualified operators are allowed to operate lift trucks.

6.0  RECORDS

Document Record Submittal Responsibility Record Retention Responsibility Safety Check – Lift Trucks

Contractor MH Construction Safety Check – Bulldozers, Crawler Tractors, Scrapers, and Other Earthmoving Equipment

Contractor MH Construction Safety Check – Miscellaneous Equipment

Contractor MH Construction Daily Crane Inspection Report

Contractor MH Construction Safety Check – Vehicle Mounted, Elevating and Rotating Aerial Devices

Contractor MH Construction

7.0  REFERENCES

ANSI A92.6	Self propelled Elevating Work Platforms ANSI A92.5	Boom Supported Elevating Work Platforms ANSI/ASME B56.1a	Safety Standards for Low Lift And High Lift Trucks ANSI/ASME B30.5	Mobile And Locomotive Cranes

8.0  ATTACHMENTS

MISH 19-A	Safety Check – Lift Trucks MISH 19-B	Safety Check – Bulldozers, Crawler Tractors, Scrapers, and Other Earthmoving Equipment MISH 19-C	Safety Check – Miscellaneous Equipment MISH 19-D	Dailey Crane Inspection Report MISH 19-E	Safety Check – Vehicle Mounted, Elevating and Rotating Aerial Devices

MH CONSTRUCTION LIFT TRUCK INSPECTION

PROJECT:___________________________PROJECT #________DATE:_________

MAKE:______________________MODEL:________________EQUIPT. #_________

TYPE:  (  )  electric     (  )  propane      (  )  gasoline     (  )  diesel   (  )______________

INSPECTOR:________________________SUPERVISOR:______________________

ITEM                                          YES    NO    N/A     DATE CORRECTED/ BY  WHOM

1) Is lift capacity prominently displayed?________________________________________ 2) Is load chart displayed and legible?_________________________________________ 3) Are warning stickers and operating control stickers clearly legible?_____________________________________________ 4) Is a fire extinguisher provided?_____________________________________________ 5) Are steps and handholds provided and in good condition for safe access?__________________________________________ 6) Are windows of safety glass or equiv. and in good, clean condition?______________________________________________ 7) Are tools and equipment secured?__________________________________________ 8) Are hydraulic lines free of leaks?___________________________________________ 9) Is vehicle free of oil or fuel leaks?___________________________________________ 10) Does horn work?________________________________________________________ 11) Does warning light work?_________________________________________________ 12) Are brakes in good working order?__________________________________________ 13) Does parking brake work properly?__________________________________________ 14) Does back-up alarm work properly?_________________________________________ 15) Are all lights in working order with no broken lenses?_______________________________________________________ 16) Do exposed moving parts have adequate guards in place?________________________________________________ 17) Are tires in good condition?________________________________________________ 18) Are fluid levels good?____________________________________________________ 19) Is Maintenance and Operations Manual with lift truck?____________________________________________________ 20) Is lift truck mechanically sound?____________________________________________ 21) Is vehicle safe to operate? If No, explain in detail.____________________________________________________

REMARKS:__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

____________________________________________________________________________

MH CONSTRUCTION EARTHMOVING EQUIPMENT INSPECTION

PROJECT:________________________PROJECT #:__________DATE:__________

TYPE OF EQUIPMENT:_________________________________________________

MAKE:______________________MODEL:__________________EQUIPT. #:_______

INSPECTOR:________________________SUPERVISOR:______________________

ITEM                                      YES   NO   N/A         DATE CORRECTED BY WHOM

1)    Is manufacturer operations and maintenance data available?____________________________________________ 2)   Are maintenance and inspection records available?_____________________________________________________ 3)   Is a fire extinguisher provided?___________________________________________ 4)   Are tools & equip. secured?_____________________________________________ 5)   Are seat belts provided?________________________________________________ 6)   Is Roll Over Protective System provided in good condition?_____________________________________________ 7)   Are windshield/windows, safety glass and in good condition?_____________________________________________ 8) Are brakes in good working order?_________________________________________ 9) Are auxiliary brakes provided & working?____________________________________________________________ 10)  Bi-directional audible alarm?_____________________________________________ 11)  Back-up audible alarm?_________________________________________________ 12)  Are all lights operable?_________________________________________________ 13)  Are moving parts guarded?______________________________________________ 14)  Are hydraulic lines in good condition?____________________________________________________________ 15)  Are lifting cables, winch lines, etc. in good operable condition?_____________________________________________ 16)  Are tires in good condition?______________________________________________ 17)  Are all fluid levels good?________________________________________________

REMARKS:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ MISC EQUIPMENT INSPECTION

INSPECTED BY:_______________________________________DATE:____________ MAKE:______________________MODEL:_____________________YEAR:________ EQUIPMENT #:______________LICENSE #:_________________EXPIRES:________ INSPECTOR:_______________________SUPERVISOR:________________________ PROJECT/DEPARTMENT:________________________________________________

ITEM                                     GOOD    BAD      N/A     DATE CORRECTED

(by whom)

HEADLIGHTS___________________________________________________________ TAILLIGHTS____________________________________________________________ TURN SIGNALS_________________________________________________________ BRAKE LIGHTS_________________________________________________________ 4-WAY FLASHERS_______________________________________________________ HORN__________________________________________________________________ BACK-UP ALARM_______________________________________________________ SEAT BELTS____________________________________________________________ WINDOW GLASS________________________________________________________ WIPERS________________________________________________________________ INTERIOR______________________________________________________________ EXTERIOR______________________________________________________________ FIRE EXTINGUISHER____________________________________________________ FIRST AID KIT__________________________________________________________ BRAKES________________________________________________________________ PARKING BRAKE_______________________________________________________ TIRES__________________________________________________________________ RUNNING CONDITION___________________________________________________ FLUID LEAKS___________________________________________________________ CLEAN & ORDERLY_____________________________________________________ GENERAL CONDITION___________________________________________________ OTHER:________________________________________________________________

LAST MAINTENANCE, if known (lube, oil, tune-up, etc.) Date:______________________

REMARKS:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ MH CONSTRUCTION MOBILE CRANE INSPECTION

PROJECT:___________________________________________PROJECT #________

SUPERVISOR/COMPETENT PERSON/INSPECTOR:___________________________DATE:_________

TYPE OF CRANE:_________________________________EQUIPMENT NUMBER:_________________ MAKE:__________________________________MODEL:______________________________________

DATE ITEM                                                YES     NO     N/A            CORRECTED  BY whom

RECORDS

MFR. Data available at jobsite:___________________________________________________________ Maintenance & repair data available:______________________________________________________ Annual inspection data available:_________________________________________________________ CAB Fire extinguisher within operators reach:___________________________________________________ Load charts/operating instructions visible:__________________________________________________ Walking surfaces clean/non-slip:__________________________________________________________ Handholds/steps provided for safe access:__________________________________________________ Safety glass windows/no breaks/clear vision:________________________________________________ All tools and equipment in cab secured:____________________________________________________

BOOMS

Boom stops provided:___________________________________________________________________ Boom angle indicator provided and operable:_______________________________________________ Boom structure free from damage/defect:__________________________________________________ Booms/jibs identified/used to MFR. Specs.:_________________________________________________

OUTRIGGERS

Means for securing when retracted/extended:_______________________________________________ Outriggers in good condition:_____________________________________________________________ Outrigger pads in good condition:_________________________________________________________

MOVING PARTS

Exposed moving parts are guarded:_______________________________________________________ Sheaves/swivels/gears/sprockets move easily:______________________________________________ Grease fittings open & free from grit:______________________________________________________ Regular lube and maintenance complete:___________________________________________________

CLUTCH & BRAKE PROTECTION

Friction brakes/clutches working(rain guards):______________________________________________

ROPES

Running ropes in good condition:_________________________________________________________

BLOCKS & HOOKS

All sheaves run smooth & true:___________________________________________________________ Swivels turn smoothly under load:_________________________________________________________ Hooks within specs & good condition:______________________________________________________ Hook safety latch in good condition:_______________________________________________________

PRIME MOVER

All lights operable/lenses intact:___________________________________________________________ Service/parking brakes in good condition:__________________________________________________ Horn/gauges/lights/controls are operable:__________________________________________________ Back-up signal provided and operable:_____________________________________________________ Seat belts provided in good condition:______________________________________________________ Windows clear/free of cracks:____________________________________________________________ Wheel chocks provided:_________________________________________________________________

HYDRAULIC & AIR HOSES

Deterioration/leakage/abrasive wear:______________________________________________________

MH CONSTRUCTION  MANLIFT INSPECTION

PROJECT:___________________________PROJECT #_________DATE:________

INSPECTED BY:________________________SUPERVISOR:___________________

MAKE:_______________________MODEL:_________________EQUIPT. #_______

ITEM                                        YES   NO  N/A       DATE CORRECTED BY WHOM 1) OPERATIONS & MAINTENANCE MANUAL AVAILABLE?______________________________________________________________________ 2) MAINTENANCE & REPAIR RECORDS AVAILABLE?______________________________________________________________________ 3) IS RATED LOAD CAPACITY POSTED?________________________________________________ 4) ARE ELECTRICAL WARNINGS POSTED?_____________________________________________ 5) ARE ALL POSTED WARNINGS LEGIBLE?_____________________________________________ 6) ALL CONTROLS LEGIBLY MARKED?________________________________________________ 7) ALL CONTROLS OPERATE PROPERLY?______________________________________________ 8) CLEAN & FREE OF DEBRIS?________________________________________________________ 9) ELECTRICAL SYSTEMS IN GOOD REPAIR?___________________________________________ 10) HYDRAULIC SYSTEMS IN GOOD REPAIR?____________________________________________ 11) GUARDRAILS IN GOOD CONDITION?________________________________________________

12) ARE ALL PARTS IN GOOD CONDITION?______________________________________________ 13) ARE MOVING PARTS SECURELY FASTENED WITH ADEQUATE GUARDS?_____________________________________________ 14) IS MEANS FOR SECURING SAFETY HARNESS PROVIDED AND IN GOOD CONDITION? (BOOMLIFT)__________________________________________________________ 15) OUTRIGGERS IN GOOD CONDITION?________________________________________________ 16) MEANS FOR SECURING OUTRIGGERS PROVIDED IN GOOD CONDITION?___________________________________________________ 17) IS FIRE EXTINGUISHER PROVIDED?_________________________________________________ 18) ARE ALL LIGHTS OPERABLE?_______________________________________________________ 19) ARE ALL GUAGES/WARNING LIGHTS IN GOOD WORKING ORDER?________________________________________________________ 20) BRAKES IN GOOD CONDITION?_____________________________________________________ REMARKS:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________