MISH 36 — Equipment Maintenance & Inspection

Chapter 5: Equipment & Vehicle Safety  |  29 CFR 1926.600  |  AGC: Hazard Control

| DOCUMENT | REVISION | EFFECTIVE DATE | APPROVED BY |
| --- | --- | --- | --- |
| MISH 36 | 3.0 | April 2026 | Jeremy Thamert |

| President | Safety Officer | Travelers Risk Control Consultant | AGC Representative |
| --- | --- | --- | --- |
| Jeremy Thamert | Matt Ramsey | _______________________ | _______________________ |
| Signature / Date | Signature / Date | Signature / Date | Signature / Date |

## ﻿Mh Construction Industrial Safety And Health Program

Title

## Equipment Maintenance And Inspection

Number

## Mish 19

Revision

1

Approved By

Jeremy Thamert                                              03/01/2025

Name                                                      Date

Effective Date

3/1/2025

Page

1 of 9

1.0  PURPOSE

To establish the system to inspect and maintain vehicles and heavy construction equipment.

2.0  RESPONSIBILITY

2.1  The Key Supervisor shall be responsible for the following:

2.1.1  Maintaining vehicles and equipment in accordance with applicable Occupational Safety and Health Administration/Washington Industrial Safety and health Act (OSHA/WISHA) and American national Standards Institute (ANSI) standards.

2.1.2  Conducting inspections of equipment, and correcting deficiencies noted during the inspections.

2.2  Employees shall be responsible for conducting daily inspections of equipment they are operating, and for reporting deficiencies to their supervisor.

3.0  DEFINITIONS

Heavy Construction Equipment:  Mobile cranes, crawler cranes, tractors, compactors, fork lifts, graders, scrapers, off-highway trucks, etc.

Motor Vehicles:  Any vehicles that operate on a construction site.

Preventive Maintenance:  Practice of utilizing periodic inspections, adjustments, lubrication, and minor repair of equipment for the purposes of extending its useful life and reducing the frequency and magnitude of repairs.

Scheduled Maintenance:  Regularly scheduled inspections, tests, lubrications, adjustments, cleaning, and tightening of various components of a piece of equipment.

4.0  GENERAL REQUIREMENTS

4.1  Equipment maintenance and inspection shall comply with the following statutory requirements:

29 Code of Federal Regulations (CFR) 1926, Subpart O; Motor Vehicles, Mechanized Equipment and Marine Operations

29 Code of Federal Regulations (CFR) 1910, Subpart N; Materials Handling and Storage

29 Code of Federal Regulations (CFR) 1910, Subpart F; Powered Platforms, Manlifts, and Vehicle-Mounted Work Platforms

Washington Administrative Code (WAS) 296-155, Part M; Motor Vehicles, Mechanized Equipment and Marine Operations.

4.2  On-site preventive maintenance of equipment and vehicles shall be performed by qualified personnel.

4.3  Motor vehicles and heavy equipment shall be maintained and inspected monthly in conformance to the manufacturer’s recommendations, and documented on equipment-specific safety check and/or inspection report forms (see attachments 19-A).

4.4  Prior to heavy equipment being brought onto a project, an inspection of the equipment shall be performed and complete the applicable safety check and inspection report forms.  The forms shall be submitted to maintenance department.  Deficiencies noted during the inspection shall be corrected prior to the equipment being brought onto the project.

4.5  MH Construction shall notify the owner or general contractor when heavy equipment is to be brought onto a project.

4.6  The key supervisor shall maintain manufacturer’s recommendations and inspection checklists onsite for review.

5.0  PROCEDURE

5.1  MOTOR VEHICLES

5.1.1  Vehicles being operated shall be checked at the beginning of each shift to ensure that the following parts, equipment, and accessories are in safe operating condition and free of apparent damage that could cause failure while in use.

Brakes (including trailer brake connections, parking brake, and emergency stopping systems.

Tires

Horn

Steering mechanism

Coupling devices

Seat belts

Operating controls

Safety devices

5.1.2  Lights, reflectors, windshield wipers, defrosters, fire extinguishers, steps, and hand-holds for vehicle access shall also be inspected where such equipment is necessary.

5.1.3  Defects noted during the motor vehicle inspection shall be corrected before the vehicle is placed into service.

5.2  HEAVY CONSTRUCTION EQUIPMENT:  The following are specific requirements relating to heavy equipment.

5.2.1  Crawler, locomotive, and truck cranes in use shall meet the applicable requirements for design, inspection, construction and testing, maintenance and operation as prescribed in the ANSI B30.5.

NOTE:	Daily inspections shall be conducted on all critical items such as brakes, crane hooks, ropes, hydraulic and pneumatic cylinders, and hydraulic and pneumatic relief pressure valves.  These inspections shall be documented and the records kept available for review.

5.2.2  Lift Trucks:  Operation of powered industrial trucks may be hazardous if maintenance is neglected or repairs, rebuilds, or adjustments are not performed in accordance with the manufacturer’s design criteria.

5.2.2.1  A scheduled, planned maintenance, lubrication, and inspection system shall be followed on all powered industrial trucks.

5.2.2.2  In an industrial truck is found to be in need or repair or is in any way unsafe or contributes to an unsafe condition, the truck shall not be operated until it has been restored to safe operating condition.

5.2.2.3  Only qualified operators are allowed to operate lift trucks.

6.0  RECORDS

Document

Record Submittal

Responsibility

Record Retention

Responsibility

Safety Check – Lift Trucks

Contractor

MH Construction

Safety Check – Bulldozers, Crawler

Tractors, Scrapers, and Other

Earthmoving Equipment

Contractor

MH Construction

Safety Check – Miscellaneous

Equipment

Contractor

MH Construction

Daily Crane Inspection Report

Contractor

MH Construction

Safety Check – Vehicle Mounted,

Elevating and Rotating Aerial Devices

Contractor

MH Construction

7.0  REFERENCES

ANSI A92.6	Self propelled Elevating Work Platforms

ANSI A92.5	Boom Supported Elevating Work Platforms

ANSI/ASME B56.1a	Safety Standards for Low Lift And High Lift Trucks

ANSI/ASME B30.5	Mobile And Locomotive Cranes

8.0  ATTACHMENTS

MISH 19-A	Safety Check – Lift Trucks

MISH 19-B	Safety Check – Bulldozers, Crawler Tractors, Scrapers, and Other Earthmoving Equipment

MISH 19-C	Safety Check – Miscellaneous Equipment

MISH 19-D	Dailey Crane Inspection Report

MISH 19-E	Safety Check – Vehicle Mounted, Elevating and Rotating Aerial Devices

## Mh Construction Lift Truck Inspection

## Project:___________________________Project #________Date:_________

## Make:______________________Model:________________Equipt. #_________

TYPE:  (  )  electric     (  )  propane      (  )  gasoline     (  )  diesel   (  )______________

## Inspector:________________________Supervisor:______________________

ITEM                                          YES    NO    N/A     DATE CORRECTED/

## By  Whom

1) Is lift capacity prominently displayed?________________________________________

2) Is load chart displayed and legible?_________________________________________

3) Are warning stickers and operating

control stickers clearly legible?_____________________________________________

4) Is a fire extinguisher provided?_____________________________________________

5) Are steps and handholds provided and

in good condition for safe access?__________________________________________

6) Are windows of safety glass or equiv.

and in good, clean condition?______________________________________________

7) Are tools and equipment secured?__________________________________________

8) Are hydraulic lines free of leaks?___________________________________________

9) Is vehicle free of oil or fuel leaks?___________________________________________

10) Does horn work?________________________________________________________

11) Does warning light work?_________________________________________________

12) Are brakes in good working order?__________________________________________

13) Does parking brake work properly?__________________________________________

14) Does back-up alarm work properly?_________________________________________

15) Are all lights in working order with

no broken lenses?_______________________________________________________

16) Do exposed moving parts have

adequate guards in place?________________________________________________

17) Are tires in good condition?________________________________________________

18) Are fluid levels good?____________________________________________________

19) Is Maintenance and Operations

Manual with lift truck?____________________________________________________

20) Is lift truck mechanically sound?____________________________________________

21) Is vehicle safe to operate?

If No, explain in detail.____________________________________________________

REMARKS:__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

____________________________________________________________________________

## Mh Construction

## Earthmoving Equipment Inspection

## Project:________________________Project #:__________Date:__________

## Type Of Equipment:_________________________________________________

## Make:______________________Model:__________________Equipt. #:_______

## Inspector:________________________Supervisor:______________________

## Item                                      Yes   No   N/A         Date Corrected

## By Whom

1)    Is manufacturer operations and

maintenance data available?____________________________________________

2)   Are maintenance and inspection

records available?_____________________________________________________

3)   Is a fire extinguisher provided?___________________________________________

4)   Are tools & equip. secured?_____________________________________________

5)   Are seat belts provided?________________________________________________

6)   Is Roll Over Protective System

provided in good condition?_____________________________________________

7)   Are windshield/windows, safety

glass and in good condition?_____________________________________________

8) Are brakes in good working order?_________________________________________

9) Are auxiliary brakes provided &

working?____________________________________________________________

10)  Bi-directional audible alarm?_____________________________________________

11)  Back-up audible alarm?_________________________________________________

12)  Are all lights operable?_________________________________________________

13)  Are moving parts guarded?______________________________________________

14)  Are hydraulic lines in good

condition?____________________________________________________________

15)  Are lifting cables, winch lines, etc.

in good operable condition?_____________________________________________

16)  Are tires in good condition?______________________________________________

17)  Are all fluid levels good?________________________________________________

REMARKS:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

## Misc Equipment Inspection

## Inspected By:_______________________________________Date:____________

## Make:______________________Model:_____________________Year:________

## Equipment #:______________License #:_________________Expires:________

## Inspector:_______________________Supervisor:________________________

## Project/Department:________________________________________________

ITEM                                     GOOD    BAD      N/A     DATE CORRECTED

(by whom)

## Headlights___________________________________________________________

## Taillights____________________________________________________________

## Turn Signals_________________________________________________________

## Brake Lights_________________________________________________________

4-WAY FLASHERS_______________________________________________________

## Horn__________________________________________________________________

## Back-Up Alarm_______________________________________________________

## Seat Belts____________________________________________________________

## Window Glass________________________________________________________

## Wipers________________________________________________________________

## Interior______________________________________________________________

## Exterior______________________________________________________________

## Fire Extinguisher____________________________________________________

## First Aid Kit__________________________________________________________

## Brakes________________________________________________________________

## Parking Brake_______________________________________________________

## Tires__________________________________________________________________

## Running Condition___________________________________________________

## Fluid Leaks___________________________________________________________

## Clean & Orderly_____________________________________________________

## General Condition___________________________________________________

## Other:________________________________________________________________

LAST MAINTENANCE, if known (lube, oil, tune-up, etc.) Date:______________________

REMARKS:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

## Mh Construction Mobile Crane Inspection

## Project:___________________________________________Project #________

## Supervisor/Competent Person/Inspector:___________________________Date:_________

TYPE OF CRANE:_________________________________EQUIPMENT NUMBER:_________________

MAKE:__________________________________MODEL:______________________________________

DATE

ITEM                                                YES     NO     N/A            CORRECTED  BY whom

## Records

MFR. Data available at jobsite:___________________________________________________________

Maintenance & repair data available:______________________________________________________

Annual inspection data available:_________________________________________________________

CAB

Fire extinguisher within operators reach:___________________________________________________

Load charts/operating instructions visible:__________________________________________________

Walking surfaces clean/non-slip:__________________________________________________________

Handholds/steps provided for safe access:__________________________________________________

Safety glass windows/no breaks/clear vision:________________________________________________

All tools and equipment in cab secured:____________________________________________________

## Booms

Boom stops provided:___________________________________________________________________

Boom angle indicator provided and operable:_______________________________________________

Boom structure free from damage/defect:__________________________________________________

Booms/jibs identified/used to MFR. Specs.:_________________________________________________

## Outriggers

Means for securing when retracted/extended:_______________________________________________

Outriggers in good condition:_____________________________________________________________

Outrigger pads in good condition:_________________________________________________________

## Moving Parts

Exposed moving parts are guarded:_______________________________________________________

Sheaves/swivels/gears/sprockets move easily:______________________________________________

Grease fittings open & free from grit:______________________________________________________

Regular lube and maintenance complete:___________________________________________________

## Clutch & Brake Protection

Friction brakes/clutches working(rain guards):______________________________________________

## Ropes

Running ropes in good condition:_________________________________________________________

## Blocks & Hooks

All sheaves run smooth & true:___________________________________________________________

Swivels turn smoothly under load:_________________________________________________________

Hooks within specs & good condition:______________________________________________________

Hook safety latch in good condition:_______________________________________________________

## Prime Mover

All lights operable/lenses intact:___________________________________________________________

Service/parking brakes in good condition:__________________________________________________

Horn/gauges/lights/controls are operable:__________________________________________________

Back-up signal provided and operable:_____________________________________________________

Seat belts provided in good condition:______________________________________________________

Windows clear/free of cracks:____________________________________________________________

Wheel chocks provided:_________________________________________________________________

## Hydraulic & Air Hoses

Deterioration/leakage/abrasive wear:______________________________________________________

## Mh Construction  Manlift Inspection

## Project:___________________________Project #_________Date:________

## Inspected By:________________________Supervisor:___________________

## Make:_______________________Model:_________________Equipt. #_______

## Item                                        Yes   No  N/A       Date Corrected

## By Whom

1) OPERATIONS & MAINTENANCE MANUAL

AVAILABLE?______________________________________________________________________

2) MAINTENANCE & REPAIR RECORDS

AVAILABLE?______________________________________________________________________

3) IS RATED LOAD CAPACITY POSTED?________________________________________________

4) ARE ELECTRICAL WARNINGS POSTED?_____________________________________________

5) ARE ALL POSTED WARNINGS LEGIBLE?_____________________________________________

6) ALL CONTROLS LEGIBLY MARKED?________________________________________________

7) ALL CONTROLS OPERATE PROPERLY?______________________________________________

8) CLEAN & FREE OF DEBRIS?________________________________________________________

9) ELECTRICAL SYSTEMS IN GOOD REPAIR?___________________________________________

10) HYDRAULIC SYSTEMS IN GOOD REPAIR?____________________________________________

11) GUARDRAILS IN GOOD CONDITION?________________________________________________

12) ARE ALL PARTS IN GOOD CONDITION?______________________________________________

13) ARE MOVING PARTS SECURELY

## Fastened With Adequate Guards?_____________________________________________

14) IS MEANS FOR SECURING SAFETY

## Harness Provided And In Good

## Condition? (Boomlift)__________________________________________________________

15) OUTRIGGERS IN GOOD CONDITION?________________________________________________

16) MEANS FOR SECURING OUTRIGGERS

## Provided In Good Condition?___________________________________________________

17) IS FIRE EXTINGUISHER PROVIDED?_________________________________________________

18) ARE ALL LIGHTS OPERABLE?_______________________________________________________

19) ARE ALL GUAGES/WARNING LIGHTS

## In Good Working Order?________________________________________________________

20) BRAKES IN GOOD CONDITION?_____________________________________________________

REMARKS:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

### Training & Reference Resources

Travelers Risk Control: https://www.travelers.com/risk-control

Travelers Training Library: https://www.travelers.com/risk-control/safety-education-training

OSHA Reference: https://www.osha.gov/construction

OSHA Reference: https://www.osha.gov/laws-regs/regulations/standardnumber/1926

AGC Safety Resources: https://www.agc.org/learn/education-training/safety-health

WISHA / WA L&I: https://lni.wa.gov/safety-health/

NOTE: WISHA citations apply only where WAC 296-155 is MORE STRINGENT than OSHA 29 CFR 1926.