MISH 49 — Incident Investigation & Root Cause Analysis

Chapter 8: Emergency Response & Incident Management  |  29 CFR 1926.20  |  AGC: Incident Investigation

| DOCUMENT | REVISION | EFFECTIVE DATE | APPROVED BY |
| --- | --- | --- | --- |
| MISH 49 | 3.0 | April 2026 | Jeremy Thamert |

| President | Safety Officer | Travelers Risk Control Consultant | AGC Representative |
| --- | --- | --- | --- |
| Jeremy Thamert | Matt Ramsey | _______________________ | _______________________ |
| Signature / Date | Signature / Date | Signature / Date | Signature / Date |

# 1.0  PURPOSE

1.1  All incidents, near-misses, and property damage events must be investigated to identify root causes and prevent recurrence.

1.2  Investigation is not about assigning blame — it is about identifying system failures and implementing corrective actions.

# 2.0  INVESTIGATION REQUIREMENTS

## 2.1  Immediate Response (0–4 Hours)

Secure the scene. Prevent further injury.

Provide first aid / call 911 if required.

Notify Safety Officer and Project Manager immediately.

Preserve physical evidence — do not disturb the scene until documented.

Conduct drug/alcohol testing per MISH 06 if required.

Notify Travelers Risk Control if incident may result in a claim.

## 2.2  Investigation (Within 24 Hours)

Photograph the scene from multiple angles.

Interview all witnesses separately. Document statements.

Collect physical evidence (equipment, materials, PPE).

Review training records, inspection logs, and work procedures.

Complete the MHC Incident Investigation Report (Form 49-A).

## 2.3  Root Cause Analysis

Use the 5-Why method to identify root causes beyond the immediate cause:

Why 1: Why did the incident occur? (Immediate cause)

Why 2: Why did that condition/action exist? (Contributing cause)

Why 3: Why was the contributing cause not prevented? (System failure)

Why 4: Why did the system failure occur? (Root cause)

Why 5: Why was the root cause not identified earlier? (Program gap)

## 2.4  Corrective Actions

2.4.1  All root causes must have a corresponding corrective action with a responsible party and a completion deadline.

2.4.2  Corrective actions must be verified as complete by the Safety Officer before the investigation is closed.

2.4.3  Lessons learned must be communicated to all affected crews at the next safety meeting.

# 3.0  RECORDKEEPING

3.1  All incident investigations must be documented on Form 49-A and retained for a minimum of 5 years.

3.2  OSHA 300/300A logs must be maintained and posted per 29 CFR 1904.

3.3  Travelers Risk Control must be notified of any incident that may result in a claim.

### Training & Reference Resources

Travelers Risk Control: https://www.travelers.com/resources/business-topics/workplace-safety/injury-impact-report

Travelers Training Library: https://www.travelers.com/risk-control/safety-education-training

OSHA Reference: https://www.osha.gov/construction

OSHA Reference: https://www.osha.gov/safety-management

AGC Safety Resources: https://www.agc.org/learn/education-training/safety-health

WISHA / WA L&I: https://lni.wa.gov/safety-health/

NOTE: WISHA citations apply only where WAC 296-155 is MORE STRINGENT than OSHA 29 CFR 1926.