MH CONSTRUCTION, INC.

Industrial Safety & Health Program (MISH)

FORM-MISH-26 | Excavation and Trenching Safety Inspection and JHA

Rev. 3 | Effective: 04/07/2026 | Safety Officer: Matt Ramsey

| Project Name / Number: |  | Job Site Address: |  |
| --- | --- | --- | --- |
| Date: |  | Supervisor / Foreman: |  |
| Employee Name(s): |  | Subcontractor: |  |

PART 1: EXCAVATION AND TRENCHING INSPECTION

Inspection Frequency: Daily prior to start of work, as needed throughout shift, and after every rainstorm or other hazard-increasing occurrence (per 29 CFR 1926.651(k)(1) and WAC 296-155-655(11)).

| Item # | Inspection Item | Pass | Fail | N/A | Corrective Action Required |
| --- | --- | --- | --- | --- | --- |
| 1 | Surface encumbrances removed or supported (29 CFR 1926.651(a)) | ☐ | ☐ | ☐ |  |
| 2 | Underground utilities located and marked (29 CFR 1926.651(b)) | ☐ | ☐ | ☐ |  |
| 3 | Access/egress provided within 25 ft for trenches >4 ft deep (29 CFR 1926.651(c)) | ☐ | ☐ | ☐ |  |
| 4 | Employees protected from vehicular traffic (29 CFR 1926.651(d)) | ☐ | ☐ | ☐ |  |
| 5 | Employees protected from falling loads (29 CFR 1926.651(e)) | ☐ | ☐ | ☐ |  |
| 6 | Warning system for mobile equipment in place (29 CFR 1926.651(f)) | ☐ | ☐ | ☐ |  |
| 7 | Hazardous atmospheres tested (>4 ft deep) (29 CFR 1926.651(g)) | ☐ | ☐ | ☐ |  |
| 8 | Emergency rescue equipment available (29 CFR 1926.651(g)(2)) | ☐ | ☐ | ☐ |  |
| 9 | Protection from water accumulation provided (29 CFR 1926.651(h)) | ☐ | ☐ | ☐ |  |
| 10 | Adjacent structures supported/protected (29 CFR 1926.651(i)) | ☐ | ☐ | ☐ |  |
| 11 | Loose rock/soil scaled or barricaded (29 CFR 1926.651(j)(1)) | ☐ | ☐ | ☐ |  |
| 12 | Spoils/materials kept >2 ft from edge (29 CFR 1926.651(j)(2)) | ☐ | ☐ | ☐ |  |
| 13 | Fall protection provided at crossings (29 CFR 1926.651(l)) | ☐ | ☐ | ☐ |  |
| 14 | Protective system designed by competent person (29 CFR 1926.652(a)) | ☐ | ☐ | ☐ |  |
| 15 | Sloping/benching meets requirements (29 CFR 1926.652(b)) | ☐ | ☐ | ☐ |  |
| 16 | Shield system (trench box) installed correctly (29 CFR 1926.652(g)) | ☐ | ☐ | ☐ |  |
| 17 | Shield system extends >18 inches above top of vertical side (29 CFR 1926.652(g)(2)) | ☐ | ☐ | ☐ |  |
| 18 | No employees in shield during installation/removal (29 CFR 1926.652(g)(1)) | ☐ | ☐ | ☐ |  |
| 19 | Competent person on site during excavation (WAC 296-155-655) | ☐ | ☐ | ☐ |  |
| 20 | Daily inspection completed by competent person (29 CFR 1926.651(k)) | ☐ | ☐ | ☐ |  |

Deficiency Log

| Item # | Description of Deficiency | Corrective Action | Assigned To | Due Date | Completed Date |
| --- | --- | --- | --- | --- | --- |

| Inspector Name: | Signature: | Date: |
| --- | --- | --- |
| Supervisor Review: | Signature: | Date: |

PART 2: JOB HAZARD ANALYSIS (JHA)

Task Description: ___________________________________________________________________________________

Pre-Task Hazard Identification:

| ☐ Cave-ins / Soil Collapse | ☐ Mobile Equipment Traffic |
| --- | --- |
| ☐ Underground Utilities | ☐ Slips, Trips, and Falls |
| ☐ Overhead Power Lines | ☐ Falling Materials/Spoils |
| ☐ Hazardous Atmosphere | ☐ Confined Space Entry |
| ☐ Water Accumulation | ☐ Noise Exposure |

Required PPE:

| ☐ Hard Hat | ☐ Work Gloves |
| --- | --- |
| ☐ Safety Glasses | ☐ Hearing Protection |
| ☐ High-Visibility Vest | ☐ Gas Monitor |
| ☐ Steel-Toe Boots | ☐ Fall Protection |

| Step # | Task Step Description | Potential Hazard | Hazard Control Measure | Responsible Party |
| --- | --- | --- | --- | --- |
| 1 | Site Preparation and Utility Locating | Striking underground utilities (gas, electric, water) | Call 811 before digging. Hand dig or pothole to verify utility locations. Maintain safe distance from marked lines. | Competent Person / Operator |
| 2 | Excavating / Trenching | Cave-in or soil collapse | Classify soil. Implement protective system (sloping, benching, shoring, or shielding) for trenches >5 ft deep. | Competent Person |
| 3 | Spoil Placement | Spoils falling back into trench | Keep spoils and equipment at least 2 feet from the edge of the excavation. | Operator / Laborers |
| 4 | Access and Egress | Slips, trips, falls; inability to exit quickly | Provide ladders, ramps, or stairs within 25 feet of travel for trenches >4 ft deep. Ensure ladder extends 3 ft above edge. | Competent Person |
| 5 | Working in Trench | Hazardous atmosphere or water accumulation | Test atmosphere if >4 ft deep and potential exists. Inspect for water accumulation daily. Use dewatering equipment if needed. | Competent Person |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

| Nearest Hospital: | Emergency Number: 911 | Site First Aid Location: |
| --- | --- | --- |

Regulatory Authority OSHA: 29 CFR 1926 Subpart P; 29 CFR 1926.651 Specific Excavation Requirements; 29 CFR 1926.652 Requirements for Protective Systems WISHA: WAC 296-155-650 Scope and Application; WAC 296-155-655 General Protection Requirements; WAC 296-155-657 Requirements for Protective Systems AGC: Safety and Health Manual Ch. 20 Excavation and Trenching This form supports compliance with the above regulations. Retain completed forms for minimum 3 years per 29 CFR 1904.33 and WAC 296-27.

| Prepared by: ________________________ | Title: ________________________ | Date: ________________________ |
| --- | --- | --- |
| Reviewed by: ________________________ | Title: ________________________ | Date: ________________________ |