MH CONSTRUCTION, INC.

Industrial Safety & Health Program (MISH)

FORM-MISH-35 | Rigging Inspection and Pre-Lift JHA

Rev. 3 | Effective: 04/07/2026 | Safety Officer: Matt Ramsey

___________________________________________________________________________

| Project Name / Number: |  | Job Site Address: |  |
| --- | --- | --- | --- |
| Date: |  | Supervisor / Foreman: |  |
| Employee Name(s): |  | Subcontractor: |  |

RIGGING INSPECTION CHECKLIST

Inspection Frequency: Prior to each shift and as necessary during use per 29 CFR 1926.251(a)(1) and WAC 296-155-33700(1).

| Item # | Inspection Item | Pass | Fail | N/A | Corrective Action Required |
| --- | --- | --- | --- | --- | --- |
| 1 | Slings - Synthetic web: No snags, punctures, tears, or cuts (29 CFR 1926.251(e)(8)) | ☐ | ☐ | ☐ |  |
| 2 | Slings - Synthetic web: No broken or worn stitches (29 CFR 1926.251(e)(8)) | ☐ | ☐ | ☐ |  |
| 3 | Slings - Synthetic web: Identification tags present and legible (29 CFR 1926.251(e)(1)) | ☐ | ☐ | ☐ |  |
| 4 | Slings - Wire rope: No excessive broken wires (29 CFR 1926.251(c)(4)) | ☐ | ☐ | ☐ |  |
| 5 | Slings - Wire rope: No severe localized abrasion or scraping (29 CFR 1926.251(c)(4)) | ☐ | ☐ | ☐ |  |
| 6 | Slings - Wire rope: No kinking, crushing, or bird caging (29 CFR 1926.251(c)(4)) | ☐ | ☐ | ☐ |  |
| 7 | Slings - Wire rope: Identification tags present and legible (29 CFR 1926.251(c)(1)) | ☐ | ☐ | ☐ |  |
| 8 | Slings - Alloy steel chain: No cracked or deformed master links/hooks (29 CFR 1926.251(b)(5)) | ☐ | ☐ | ☐ |  |
| 9 | Slings - Alloy steel chain: No excessive wear or stretching (29 CFR 1926.251(b)(5)) | ☐ | ☐ | ☐ |  |
| 10 | Slings - Alloy steel chain: Identification tags present and legible (29 CFR 1926.251(b)(1)) | ☐ | ☐ | ☐ |  |
| 11 | Shackles: No deformation, cracks, or excessive wear (29 CFR 1926.251(f)(1)) | ☐ | ☐ | ☐ |  |
| 12 | Shackles: Pins are correct type and properly seated (29 CFR 1926.251(f)(1)) | ☐ | ☐ | ☐ |  |
| 13 | Hooks: No cracks, nicks, or gouges (29 CFR 1926.251(a)(1)) | ☐ | ☐ | ☐ |  |
| 14 | Hooks: Throat opening not increased by more than 15% (29 CFR 1926.251(a)(1)) | ☐ | ☐ | ☐ |  |
| 15 | Hooks: Safety latches present and functioning properly (WAC 296-155-33810) | ☐ | ☐ | ☐ |  |
| 16 | Eyebolts/Rings: No cracks, bends, or elongation (29 CFR 1926.251(a)(1)) | ☐ | ☐ | ☐ |  |
| 17 | Custom Lifting Devices: Marked with capacity and weight (29 CFR 1926.251(a)(4)) | ☐ | ☐ | ☐ |  |
| 18 | Custom Lifting Devices: Structurally sound, no visible defects (29 CFR 1926.251(a)(1)) | ☐ | ☐ | ☐ |  |
| 19 | Rigging Hardware: Free of excessive rust or corrosion (29 CFR 1926.251(a)(1)) | ☐ | ☐ | ☐ |  |
| 20 | Storage: Rigging stored properly off the ground and protected (WAC 296-155-33700) | ☐ | ☐ | ☐ |  |

DEFICIENCY LOG

| Item # | Description of Deficiency | Corrective Action | Assigned To | Due Date | Completed Date |
| --- | --- | --- | --- | --- | --- |

| Inspector Name: ____________________ | Signature: ____________________ | Date: ____________ |
| --- | --- | --- |
| Supervisor Review: _________________ | Signature: ____________________ | Date: ____________ |

PRE-LIFT JOB HAZARD ANALYSIS (JHA)

Task Description: ___________________________________________________________________________________

Pre-Task Hazard Identification:

| ☐ Overhead Power Lines | ☐ Wind/Weather Conditions | ☐ Uneven/Soft Ground |
| --- | --- | --- |
| ☐ Blind Lifts/Poor Visibility | ☐ Swing Radius Hazards | ☐ Traffic/Pedestrians |
| ☐ Load Instability | ☐ Sharp Edges on Load | ☐ Pinch Points |
| ☐ Communication Issues | ☐ Equipment Failure | ☐ Suspended Loads |

Required PPE:

| ☐ Hard Hat | ☐ Safety Glasses | ☐ High-Vis Apparel |
| --- | --- | --- |
| ☐ Steel-Toe Boots | ☐ Leather Gloves | ☐ Fall Protection |
| ☐ Hearing Protection | ☐ Taglines |  |

| Step # | Task Step Description | Potential Hazard | Hazard Control Measure | Responsible Party |
| --- | --- | --- | --- | --- |
| 1 | Inspect rigging equipment | Using damaged rigging causing load drop | Perform pre-use inspection per checklist. Remove damaged rigging from service. | Rigger |
| 2 | Attach rigging to load | Pinch points, sharp edges cutting slings | Keep hands clear of pinch points. Use softeners/edge protection on sharp corners. | Rigger |
| 3 | Verify load weight and center of gravity | Overloading crane, load shifting/tipping | Confirm weight with documentation. Perform test lift to verify balance. | Lift Director / Rigger |
| 4 | Lift and move load | Struck by load, swing radius hazards | Use taglines to control load. Keep personnel out of fall zone and swing radius. | Operator / Rigger |
| 5 | Communicate during lift | Misunderstanding signals causing accident | Establish clear communication method (radio or standard hand signals). Designate one signal person. | Signal Person / Operator |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

| Nearest Hospital: ____________________ | Emergency Number: 911 | Site First Aid Location: _______________ |
| --- | --- | --- |

Regulatory Authority OSHA: 29 CFR 1926 Subpart CC; 29 CFR 1926.251 Rigging Equipment for Material Handling WISHA: WAC 296-155-336 through 296-155-340 Rigging Requirements for Material Handling; WAC 296-155-556 through 296-155-562 Crane Rigging Requirements AGC: Safety and Health Manual Ch. 15 Cranes and Rigging This form supports compliance with the above regulations. Retain completed forms for minimum 3 years per 29 CFR 1904.33 and WAC 296-27.

| Prepared by: ________________________ | Title: ________________________ | Date: ________________ |
| --- | --- | --- |
| Reviewed by: ________________________ | Title: ________________________ | Date: ________________ |