CONTROLLED ENTERPRISE DOCUMENT TBT-01.02 ¦ Respiratory Protection MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-01.02 ¦ Respiratory Protection Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-01.02: Respiratory Protection

CSI Division: Div 01 | OSHA Reference: 29 CFR 1926.103

Delivery: Mandatory before any work requiring a respirator, and annually thereafter.

## DIRECTIVE — Policy Standard

No employee shall be required or permitted to wear a respirator unless they have been medically cleared, fit-tested, and trained in accordance with the MH Construction, INC Respiratory Protection Program.

### GENERAL ORDER — Procedure

The Safety Officer manages medical evaluations and fit testing. Foremen must ensure that only authorized respirators are used, that they are inspected before each use, and that they are stored in a clean, sanitary location.

### Key Hazards & Mitigation (The 'What If'):

• Dust Mask vs. Respirator: If a worker brings their own N95 mask from home for comfort, they must sign a voluntary use form. If MH requires an N95 for the task, full medical clearance and fit testing apply.

• Facial Hair: If a worker arrives with a beard that interferes with the respirator seal, they cannot perform the work requiring the respirator. Reassign them or send them home to shave.

• Cartridge Expiration: If a worker is using a half-mask with organic vapor cartridges, the Foreman must enforce the change-out schedule. Do not wait until the worker smells the chemical.

#### FIELD INSTRUCTION — Execution

FIELD INSTRUCTION: Foremen must verify fit-test cards for all crew members requiring respirators today. Log attendance on MISH-F-10.1.

Attendance Roster (Log on MISH-F-10.1):

| Print Name | Signature | Company/Trade |
| --- | --- | --- |