CONTROLLED ENTERPRISE DOCUMENT TBT-01.05 ¦ Fall Protection And Elevated Work MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-01.05 ¦ Fall Protection And Elevated Work Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-01.05: Fall Protection & Elevated Work

CSI Division: Div 01 | OSHA Reference: 29 CFR 1926.503

Delivery: Mandatory before any work at heights of 6 feet or greater.

## DIRECTIVE — Policy Standard

100% continuous fall protection is required for all personnel working at heights of 6 feet or greater above a lower level.

### GENERAL ORDER — Procedure

Personal Fall Arrest Systems (PFAS) must be inspected before each use. Anchor points must be capable of supporting 5,000 lbs per employee attached. Lanyards must be calculated to prevent the worker from contacting the lower level.

### Key Hazards & Mitigation (The 'What If'):

• Leading Edge Work: If a worker is approaching an unprotected edge, they must be tied off before entering the fall hazard zone. Use a self-retracting lifeline (SRL) rated for leading edges.

• Suspension Trauma: If a worker falls and is suspended in their harness, rescue must occur within 15 minutes. Deploy the site-specific rescue plan immediately.

• Improper Tie-Off: If a worker ties off to a guardrail, plumbing pipe, or electrical conduit, stop work immediately. These are not rated anchor points.

#### FIELD INSTRUCTION — Execution

FIELD INSTRUCTION: Foremen must physically inspect all harnesses and lanyards in use today. Verify anchor points and rescue plans. Log attendance on MISH-F-10.1 and complete MISH-F-21.1.

Attendance Roster (Log on MISH-F-10.1):

| Print Name | Signature | Company/Trade |
| --- | --- | --- |