CONTROLLED ENTERPRISE DOCUMENT TBT-03.03 ¦ Concrete Pump Truck Operations MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-03.03 ¦ Concrete Pump Truck Operations Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-03.03: Concrete Pump Truck Operations

CSI Division: Div 03 | Track Assignment: Track 1 Civil & Earthwork

## DIRECTIVE

Pump truck outriggers must be fully extended and placed on stable ground or dunnage pads. Stay clear of the pump boom's swing radius.

## GENERAL ORDER

Never stand between the pump truck and the ready-mix truck. The hose handler must maintain clear communication with the operator.

## Key Hazards & Mitigation (The 'What If')

- Hose whips unexpectedly — keep a firm grip, do not straddle the hose.

- Outrigger sinks into soft dirt — stop pumping, reset on solid dunnage.

- Concrete splatter to the face — wear safety glasses and face shield.

## FIELD INSTRUCTION

Foreman: Verify outrigger placement and ground stability before the first truck backs in.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |

Toolbox Talk Documentation & Sign-Off
This Toolbox Talk (TBT) is a mandatory component of the MH Construction, INC Industrial Safety & Health (MISH) Program. All site personnel, including subcontractors and temporary laborers, must attend this briefing before commencing related work activities. 

The Site Safety Commander or designated Foreman is responsible for delivering this content, ensuring comprehension, and documenting attendance. Failure to adhere to the safety protocols outlined herein may result in disciplinary action up to and including removal from the jobsite.

By signing the daily attendance log (Form MISH-F-10.1), you acknowledge that you have received, understood, and agree to comply with the safety directives presented in this Toolbox Talk.