CONTROLLED ENTERPRISE DOCUMENT TBT-05.04 ¦ Metal Decking And Controlled Access Zones MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-05.04 ¦ Metal Decking And Controlled Access Zones Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-05.04: Metal Decking & Controlled Access Zones

CSI Division: Div 05 | Track Assignment: Track 2 Structural & Steel

## DIRECTIVE

Metal decking operations require a Controlled Decking Zone (CDZ). Only workers engaged in leading-edge work are permitted in the CDZ.

## GENERAL ORDER

Decking must be laid tightly and immediately secured to prevent sliding. Do not use unsecured decking as a walking surface.

## Key Hazards & Mitigation (The 'What If')

- Unsecured sheet slips under a worker — never step on a sheet until it is tacked or screwed down.

- Other trades enter the CDZ — halt work and clear unauthorized personnel from the zone.

- Wind picks up during decking — secure all loose sheets immediately and assess if lifting should stop.

## FIELD INSTRUCTION

Foreman: Establish the CDZ with warning lines and restrict access to the decking crew only.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |

Toolbox Talk Documentation & Sign-Off
This Toolbox Talk (TBT) is a mandatory component of the MH Construction, INC Industrial Safety & Health (MISH) Program. All site personnel, including subcontractors and temporary laborers, must attend this briefing before commencing related work activities. 

The Site Safety Commander or designated Foreman is responsible for delivering this content, ensuring comprehension, and documenting attendance. Failure to adhere to the safety protocols outlined herein may result in disciplinary action up to and including removal from the jobsite.

By signing the daily attendance log (Form MISH-F-10.1), you acknowledge that you have received, understood, and agree to comply with the safety directives presented in this Toolbox Talk.