CONTROLLED ENTERPRISE DOCUMENT TBT-06.01 ¦ Rough Carpentry And Pneumatic Tools MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-06.01 ¦ Rough Carpentry And Pneumatic Tools Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-06.01: Rough Carpentry & Pneumatic Tools

CSI Division: Div 06 | Track Assignment: Track 2 Structural & Steel

## DIRECTIVE

Pneumatic nail guns must have a safety device on the muzzle to prevent the tool from ejecting fasteners unless the muzzle is in contact with the work surface.

## GENERAL ORDER

Never bypass or tape down the safety nose contact. Disconnect the air hose before clearing a jam or performing maintenance.

## Key Hazards & Mitigation (The 'What If')

- Nail gun double-fires — disconnect air, inspect safety mechanism.

- Worker holding wood behind the nailing surface — reposition hands, use clamps.

- Hose trips worker on framing deck — route hoses safely, use hose hooks.

## FIELD INSTRUCTION

Foreman: Inspect all pneumatic tools for bypassed safeties before use. Confiscate altered tools immediately.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |

Toolbox Talk Documentation & Sign-Off
This Toolbox Talk (TBT) is a mandatory component of the MH Construction, INC Industrial Safety & Health (MISH) Program. All site personnel, including subcontractors and temporary laborers, must attend this briefing before commencing related work activities. 

The Site Safety Commander or designated Foreman is responsible for delivering this content, ensuring comprehension, and documenting attendance. Failure to adhere to the safety protocols outlined herein may result in disciplinary action up to and including removal from the jobsite.

By signing the daily attendance log (Form MISH-F-10.1), you acknowledge that you have received, understood, and agree to comply with the safety directives presented in this Toolbox Talk.