CONTROLLED ENTERPRISE DOCUMENT TBT-09.03 ¦ Painting Scaffolds And Chemical Storage MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-09.03 ¦ Painting Scaffolds And Chemical Storage Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-09.03: Painting Scaffolds & Chemical Storage

CSI Division: Div 09 | Track Assignment: Track 3 MEP & Interior

## DIRECTIVE

Baker scaffolds used for painting must have all wheels locked before climbing. Paint and solvents must be stored in approved flammable cabinets at the end of the shift.

## GENERAL ORDER

Do not overreach from the scaffold—move the scaffold instead. Clean up all spills immediately to prevent slip hazards.

## Key Hazards & Mitigation (The 'What If')

- Worker reaches too far and tips the baker scaffold — keep your belt buckle within the scaffold rails.

- Oily rags left in a pile — dispose of in a designated, self-closing metal safety can to prevent spontaneous combustion.

- Scaffold wheels unlocked while painting — stop work, lock all four casters before ascending.

## FIELD INSTRUCTION

Foreman: Inspect the flammable storage cabinet at the end of the shift to ensure all solvents and paints are secured.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |