CONTROLLED ENTERPRISE DOCUMENT TBT-10.01 ¦ Installation Of Heavy Specialties Lockers Partitions MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-10.01 ¦ Installation Of Heavy Specialties Lockers Partitions Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-10.01: Installation of Heavy Specialties (Lockers/Partitions)

CSI Division: Div 10 | Track Assignment: Track 3 MEP & Interior

## DIRECTIVE

Heavy specialty items must be securely anchored to structural members, not just drywall, to prevent tip-over hazards.

## GENERAL ORDER

Use the correct anchors for the substrate (concrete, steel, wood). Two-person lift required for any specialty item over 50 lbs.

## Key Hazards & Mitigation (The 'What If')

- Locker bank tips forward during install — clear the area, use temporary wall braces.

- Drilling into wall hits resistance — stop, check for conduit or pipes.

- Awkward lift in tight bathroom — use material lift or extra personnel.

## FIELD INSTRUCTION

Foreman: Verify the anchor specifications match the substrate before installation begins.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |

Toolbox Talk Documentation & Sign-Off
This Toolbox Talk (TBT) is a mandatory component of the MH Construction, INC Industrial Safety & Health (MISH) Program. All site personnel, including subcontractors and temporary laborers, must attend this briefing before commencing related work activities. 

The Site Safety Commander or designated Foreman is responsible for delivering this content, ensuring comprehension, and documenting attendance. Failure to adhere to the safety protocols outlined herein may result in disciplinary action up to and including removal from the jobsite.

By signing the daily attendance log (Form MISH-F-10.1), you acknowledge that you have received, understood, and agree to comply with the safety directives presented in this Toolbox Talk.