CONTROLLED ENTERPRISE DOCUMENT TBT-11.01 ¦ Ariel Lifts Mewp And Fall Restraint MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-11.01 ¦ Ariel Lifts Mewp And Fall Restraint Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-11.01: Ariel Lifts (MEWP) & Fall Restraint

CSI Division: Div 11 | OSHA Reference: 29 CFR 1926.453

Delivery: Mandatory for all personnel operating boom lifts or scissor lifts.

## DIRECTIVE — Policy Standard

Personnel in an articulating or telescoping boom lift must wear a full-body harness with a fall restraint lanyard attached to the manufacturer's engineered anchor point in the basket.

### GENERAL ORDER — Procedure

Lifts must only be operated on firm, level surfaces. Driving a boom lift with the platform elevated is strictly prohibited unless specifically permitted by the manufacturer's manual for that exact model.

### Key Hazards & Mitigation (The 'What If'):

• Climbing the Rails: If a worker needs 'just two more inches' of reach and steps on the mid-rail of the basket, they have defeated the fall protection system. Keep both feet on the floor of the basket.

• Catapult Effect: If a boom lift drives over a 2-inch curb while elevated, the basket will whip violently, potentially catapulting the worker out. Always lower the boom before moving.

• Crush Hazard (Entrapment): If operating a lift in tight overhead steel, the operator must look up continuously. If the operator is pinned against a beam, the ground controls must be used for rescue.

#### FIELD INSTRUCTION — Execution

FIELD INSTRUCTION: Foremen must verify that all lift operators have their certification cards and have completed the daily pre-use inspection before elevating. Log attendance on MISH-F-10.1.

Attendance Roster (Log on MISH-F-10.1):

| Print Name | Signature | Company/Trade |
| --- | --- | --- |