CONTROLLED ENTERPRISE DOCUMENT TBT-11.04 ¦ Warehouse Forklift Operations MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-11.04 ¦ Warehouse Forklift Operations Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-11.04: Warehouse Forklift Operations & Pedestrian Traffic

CSI Division: Div 11 | OSHA Reference: 29 CFR 1910.178

Delivery: Mandatory for all warehouse staff, mechanics, and personnel entering the MH yard.

## DIRECTIVE — Policy Standard

Forklift operators must yield the right-of-way to pedestrians at all times, sound their horn at blind intersections, and travel with the load trailing if forward vision is obstructed.

### GENERAL ORDER — Procedure

Pedestrians in the warehouse or yard must stay within designated walkways and wear high-visibility vests. Never walk under a suspended load, even if it is 'just for a second.'

### Key Hazards & Mitigation (The 'What If'):

• Blind Corners: If a forklift is approaching the end of a storage rack aisle, the operator must stop, sound the horn, and proceed slowly. Pedestrians must stop and look before crossing aisles.

• Unsecured Loads: If a pallet of material is shrink-wrapped poorly and leaning, the operator must lower the load and re-wrap it before moving. Do not attempt to 'balance' it while driving.

• Unauthorized Operators: If a Foreman needs a pallet moved but the certified warehouse operator is on break, the Foreman cannot jump on the forklift unless they carry a current MH certification card.

#### FIELD INSTRUCTION — Execution

FIELD INSTRUCTION: Warehouse Manager must verify EOM-F-02.1 (Operator Certification Log) for all active operators today. Log attendance on MISH-F-10.1.

Attendance Roster (Log on MISH-F-10.1):

| Print Name | Signature | Company/Trade |
| --- | --- | --- |