CONTROLLED ENTERPRISE DOCUMENT TBT-11.05 ¦ Manual Material Handling And Ergonomics MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-11.05 ¦ Manual Material Handling And Ergonomics Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-11.05: Manual Material Handling & Ergonomics

CSI Division: Div 11 | OSHA Reference: 29 CFR 1926.20

Delivery: Mandatory for all crews involved in manual loading, unloading, or staging of materials.

## DIRECTIVE — Policy Standard

Employees must assess the weight and shape of a load before lifting. Any load exceeding 50 lbs requires a two-person team lift or mechanical assistance (dolly, pallet jack, forklift).

### GENERAL ORDER — Procedure

When lifting, keep the load close to the body, bend at the knees, and lift with the legs—never the back. Do not twist the torso while carrying a load; pivot with your feet.

### Key Hazards & Mitigation (The 'What If'):

• Awkward Shapes: If a worker needs to move a 40 lb piece of ductwork that is 10 feet long, the awkward shape makes it a two-person lift despite being under the 50 lb limit.

• Repetitive Stacking: If a crew is hand-unloading a truck of block, rotate tasks every hour to prevent fatigue. Use a conveyor or staging table to keep the work between knee and shoulder height.

• Dropped Loads: If a heavy load slips from your grip, let it fall. Do not attempt to catch it awkwardly, which causes severe back and shoulder injuries. Step back and let it drop.

#### FIELD INSTRUCTION — Execution

FIELD INSTRUCTION: Foremen must identify all heavy lifts scheduled for today's shift and assign team lifts or mechanical aids before work begins. Log attendance on MISH-F-10.1.

Attendance Roster (Log on MISH-F-10.1):

| Print Name | Signature | Company/Trade |
| --- | --- | --- |