CONTROLLED ENTERPRISE DOCUMENT TBT-14.01 ¦ Crane Operations And Critical Lifts MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-14.01 ¦ Crane Operations And Critical Lifts Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-14.01: Crane Operations & Critical Lifts

CSI Division: Div 14 | OSHA Reference: 29 CFR 1926.1400

Delivery: Mandatory before any crane mobilization or lift exceeding 75% of capacity.

## DIRECTIVE — Policy Standard

All crane lifts must be planned and executed under the direction of a designated Lift Director. Any lift exceeding 75% of the crane's capacity, or involving multiple cranes, is a Critical Lift requiring an approved MISH-F-38.1 plan.

### GENERAL ORDER — Procedure

The crane must be set up on firm, level ground with fully extended outriggers on engineered pads. Only one designated rigger/signal person may communicate with the operator during the lift.

### Key Hazards & Mitigation (The 'What If'):

• Wind Speed Limits: If sustained winds exceed 20 mph, or the crane manufacturer's limit, the lift must be suspended. The operator has final stop-work authority.

• Outrigger Sinking: If the crane is set up near an excavation and an outrigger pad begins to sink into the soil, stop the lift immediately, lower the load, and reposition the crane.

• Multiple Signalers: If three different workers are shouting instructions or giving hand signals to the operator, the operator must stop all movement until the single designated signaler is identified.

#### FIELD INSTRUCTION — Execution

FIELD INSTRUCTION: Foremen must verify the crane operator's NCCCO certification and review the MISH-F-38.1 Lift Plan before the crane hooks the first load. Log attendance on MISH-F-10.1.

Attendance Roster (Log on MISH-F-10.1):

| Print Name | Signature | Company/Trade |
| --- | --- | --- |