CONTROLLED ENTERPRISE DOCUMENT TBT-22.02 ¦ Pipe Testing And Pressurization MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-22.02 ¦ Pipe Testing And Pressurization Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-22.02: Pipe Testing & Pressurization

CSI Division: Div 22 | OSHA Reference: 29 CFR 1926.302

Delivery: Mandatory for plumbing and mechanical crews conducting pressure tests.

## DIRECTIVE — Policy Standard

Pneumatic (air) testing of PVC or CPVC pipe is strictly prohibited due to the risk of explosive fragmentation. Hydrostatic (water) testing is the required method for plastic piping.

### GENERAL ORDER — Procedure

During any pressure test (air or water), a danger zone must be established. Non-essential personnel must be cleared from the area, and pressure must be increased in slow, controlled increments.

### Key Hazards & Mitigation (The 'What If'):

• Cap Blowout: If a test cap fails at 150 PSI, it becomes a lethal projectile. Never stand directly in front of a test cap or valve while the system is under pressure.

• Pneumatic Steel Test: If testing steel pipe with air, the stored energy is massive. If a weld fails, the shockwave can cause severe acoustic trauma and structural damage. Clear the floor.

• Bleeding Pressure: If the test is complete, the pressure must be bled off slowly through a designated relief valve. Never loosen a fitting to 'crack' the pressure.

#### FIELD INSTRUCTION — Execution

FIELD INSTRUCTION: Foremen must review the test plan, verify the gauge calibration, and establish the barricade zone before turning on the pump/compressor. Log attendance on MISH-F-10.1.

Attendance Roster (Log on MISH-F-10.1):

| Print Name | Signature | Company/Trade |
| --- | --- | --- |