CONTROLLED ENTERPRISE DOCUMENT TBT-22.04 ¦ Cast Iron Pipe Cutting And Snapping MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-22.04 ¦ Cast Iron Pipe Cutting And Snapping Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-22.04: Cast Iron Pipe Cutting & Snapping

CSI Division: Div 22 | Track Assignment: Track 3 MEP & Interior

## DIRECTIVE

Cutting cast iron pipe creates sharp shards and requires significant force. Snap cutters must be maintained and used with proper PPE.

## GENERAL ORDER

Wear heavy leather gloves and a full face shield when snapping cast iron. Ensure the pipe is fully supported on both sides of the cut.

## Key Hazards & Mitigation (The 'What If')

- Pipe snaps unevenly and drops — support the pipe with blocks or a stand.

- Metal shard flies into the eye — wear a face shield, not just safety glasses.

- Worker pinches fingers in the snap cutter chain — keep hands clear of the chain mechanism during the snap.

## FIELD INSTRUCTION

Foreman: Inspect the snap cutter chain for dull or missing wheels before cutting operations begin.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |

Toolbox Talk Documentation & Sign-Off
This Toolbox Talk (TBT) is a mandatory component of the MH Construction, INC Industrial Safety & Health (MISH) Program. All site personnel, including subcontractors and temporary laborers, must attend this briefing before commencing related work activities. 

The Site Safety Commander or designated Foreman is responsible for delivering this content, ensuring comprehension, and documenting attendance. Failure to adhere to the safety protocols outlined herein may result in disciplinary action up to and including removal from the jobsite.

By signing the daily attendance log (Form MISH-F-10.1), you acknowledge that you have received, understood, and agree to comply with the safety directives presented in this Toolbox Talk.