CONTROLLED ENTERPRISE DOCUMENT TBT-23.03 ¦ Refrigerant Handling And Brazing MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-23.03 ¦ Refrigerant Handling And Brazing Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-23.03: Refrigerant Handling & Brazing

CSI Division: Div 23 | Track Assignment: Track 3 MEP & Interior

## DIRECTIVE

Refrigerant gases displace oxygen and can cause asphyxiation in confined spaces. Brazing lines that previously contained refrigerant can create toxic phosgene gas.

## GENERAL ORDER

Purge lines with nitrogen before brazing. Never braze a line that is under pressure or contains residual refrigerant.

## Key Hazards & Mitigation (The 'What If')

- Worker feels dizzy in a mechanical room — evacuate immediately, ventilate the space.

- Flame turns green while brazing — stop immediately, phosgene gas is present, purge the line thoroughly.

- Cylinder valve is damaged — do not use, tag out and return to supplier.

## FIELD INSTRUCTION

Foreman: Verify the nitrogen purge is active before any brazing begins on existing lines.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |

Toolbox Talk Documentation & Sign-Off
This Toolbox Talk (TBT) is a mandatory component of the MH Construction, INC Industrial Safety & Health (MISH) Program. All site personnel, including subcontractors and temporary laborers, must attend this briefing before commencing related work activities. 

The Site Safety Commander or designated Foreman is responsible for delivering this content, ensuring comprehension, and documenting attendance. Failure to adhere to the safety protocols outlined herein may result in disciplinary action up to and including removal from the jobsite.

By signing the daily attendance log (Form MISH-F-10.1), you acknowledge that you have received, understood, and agree to comply with the safety directives presented in this Toolbox Talk.