CONTROLLED ENTERPRISE DOCUMENT TBT-33.01 ¦ Deep Sewer And Water Main Trenching MH Construction, INC Enterprise Management System Document family: TOOLBOX TALK LIBRARY Document title: TBT-33.01 ¦ Deep Sewer And Water Main Trenching Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# TBT-33.01: Deep Sewer & Water Main Trenching

CSI Division: Div 33 | Track Assignment: Track 1 Civil & Earthwork

## DIRECTIVE

All trenches over 5 feet deep require a trench box or proper sloping. A Competent Person must inspect the trench daily.

## GENERAL ORDER

Keep excavated spoil piles at least 2 feet back from the edge of the trench. Provide safe access/egress within 25 feet of workers.

## Key Hazards & Mitigation (The 'What If')

- Trench wall begins to slough — evacuate immediately.

- Worker enters trench without box — stop work, enforce trench safety rules.

- Heavy equipment operating too close to edge — move equipment back to prevent collapse.

## FIELD INSTRUCTION

Foreman: The Competent Person must sign off on the trench inspection before anyone enters the excavation.

## Attendance Roster (MISH-F-10.1)

| Print Name | Signature | Date |
| --- | --- | --- |

Toolbox Talk Documentation & Sign-Off
This Toolbox Talk (TBT) is a mandatory component of the MH Construction, INC Industrial Safety & Health (MISH) Program. All site personnel, including subcontractors and temporary laborers, must attend this briefing before commencing related work activities. 

The Site Safety Commander or designated Foreman is responsible for delivering this content, ensuring comprehension, and documenting attendance. Failure to adhere to the safety protocols outlined herein may result in disciplinary action up to and including removal from the jobsite.

By signing the daily attendance log (Form MISH-F-10.1), you acknowledge that you have received, understood, and agree to comply with the safety directives presented in this Toolbox Talk.