CONTROLLED ENTERPRISE DOCUMENT SDS-QR-06 ¦ Lead Paint MH Construction, INC Enterprise Management System Document family: SDS LIBRARY Document title: SDS-QR-06 ¦ Lead Paint Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# SDS Quick Reference: Lead-Based Paint (LBP) Dust/Debris

Manufacturer: Pre-1978 existing structures

## DIRECTIVE — Hazard Identification

DANGER: May damage fertility or the unborn child. May cause harm to breast-fed children. Causes damage to central nervous system, blood forming organs, and kidneys through prolonged or repeated exposure.

### GENERAL ORDER — First Aid Measures

• INHALATION: Move to fresh air. Seek medical evaluation for blood lead level testing.

• INGESTION: Seek medical attention immediately. Do not induce vomiting unless directed by medical personnel.

• SKIN/EYES: Wash thoroughly with lead-specific soap. Flush eyes with water.

#### FIELD INSTRUCTION — Required PPE

FIELD INSTRUCTION: Wear half-mask respirator with P100 filters, disposable coveralls, and gloves during abatement or demolition. Do not eat, drink, or smoke in the work area.