CONTROLLED ENTERPRISE DOCUMENT SDS-QR-08 ¦ Asbestos Acm MH Construction, INC Enterprise Management System Document family: SDS LIBRARY Document title: SDS-QR-08 ¦ Asbestos Acm Version: 1.0 ¦ Lifecycle: Rough Draft Use only after required functional, technical, field-usability, and approval gates are complete.

Control: Version 1.0 | Status Rough Draft | Author and Chief Editor: Matt Ramsey | Co-Editor: Todd Shoeff | Final Approver: Jeremy Thamert

# SDS Quick Reference: Asbestos-Containing Material (ACM)

Manufacturer: Pre-1981 insulation, flooring, roofing

## DIRECTIVE — Hazard Identification

DANGER: May cause cancer (Mesothelioma, Lung Cancer). Causes damage to lungs through prolonged or repeated exposure (Asbestosis). Inhalation hazard.

### GENERAL ORDER — First Aid Measures

• INHALATION: Move to fresh air. Seek medical evaluation.

• SKIN/EYES: Wash thoroughly with water. Remove contaminated clothing in a designated decontamination area.

• INGESTION: Unlikely route of exposure. Seek medical advice.

#### FIELD INSTRUCTION — Required PPE

FIELD INSTRUCTION: Only certified asbestos abatement workers may handle ACM. Wear full-face supplied-air respirator (SAR) or powered air-purifying respirator (PAPR) with HEPA filters, and disposable Tyvek suits.