MH CONSTRUCTION, INC.

Industrial Safety & Health Program (MISH)

FORM-MISH-21 | Crane Suspended Work Platform Permit and JHA

Rev. 3 | Effective: 04/07/2026 | Safety Officer: Matt Ramsey

| Project Name / Number: |  | Job Site Address: |  |
| --- | --- | --- | --- |
| Date: |  | Supervisor / Foreman: |  |
| Employee Name(s): |  | Subcontractor: |  |

PART 1: WORK AUTHORIZATION PERMIT

Permit Number: [DATE]-[MISH21]-[SEQ]

Pre-work Conditions Checklist:

☐ Trial lift performed and documented

☐ Proof testing of platform completed

☐ Crane set up on firm, level footing

☐ Outriggers fully extended and blocked

☐ Wind speed verified within safe limits (<20 mph)

☐ Load capacity verified (platform + personnel + tools < 50% of crane capacity)

☐ Anti-two-block device operational

☐ Controlled load lowering capability verified

☐ Positive locking hooks on all rigging

☐ Communication system established between operator and platform

☐ Fall protection systems inspected and ready

☐ Pre-lift meeting conducted with all involved personnel

Work Authorization:

| Work Description | Location | Start Time | End Time | Authorized By |
| --- | --- | --- | --- | --- |

Required Controls Checklist:

☐ Personal Fall Arrest Systems (PFAS) required for all occupants

☐ Lanyards attached to designated anchor points only

☐ Hard hats with chin straps required

☐ Tools and materials secured from falling

☐ Barricades erected below lift area

☐ Tag lines used to control platform

PART 2: JOB HAZARD ANALYSIS (JHA)

Task Description: ______________________________________________________________________

Pre-task Hazard Identification:

| ☐ Fall from elevation | ☐ Pinch points / crushing |
| --- | --- |
| ☐ Struck by falling objects | ☐ Weather hazards (wind, lightning) |
| ☐ Crane tip-over | ☐ Rigging failure |
| ☐ Power line contact | ☐ Communication failure |

Required PPE:

| ☐ Hard Hat | ☐ Full Body Harness |
| --- | --- |
| ☐ Safety Glasses | ☐ Shock-absorbing Lanyard |
| ☐ High-Vis Vest | ☐ Work Gloves |
| ☐ Steel-toe Boots | ☐ Hearing Protection |

| Step # | Task Step Description | Potential Hazard | Hazard Control Measure | Responsible Party |
| --- | --- | --- | --- | --- |
| 1 | Crane Setup | Tip-over, uneven ground | Verify firm footing, outriggers fully extended and blocked | Operator |
| 2 | Rigging Platform | Rigging failure, dropped platform | Inspect all rigging, use positive locking hooks, shackle pins secured | Rigger |
| 3 | Trial Lift | Platform instability, interference | Perform trial lift with anticipated weight, check clearances | Operator / Rigger |
| 4 | Personnel Boarding | Slips, trips, falls | Board only when platform is grounded and stable, tie-off immediately | All Personnel |
| 5 | Hoisting Personnel | Falls, struck by, contact with structures | Maintain 100% tie-off, clear communication, smooth operation, tag lines | Operator / Occupants |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

EMERGENCY CONTACTS: Nearest Hospital: ____________________ ¦ Emergency Number: 911 ¦ Site First Aid Location: ____________________

Permit Closure:

☐ Work completed    ☐ Area restored

Permit closed by: ________________________    Signature: ________________________    Date/Time: ________________

Regulatory Authority OSHA: 29 CFR 1926 Subpart CC; 29 CFR 1926.1431 Hoisting Personnel WISHA: WAC 296-155-547 Personnel Lifting Platforms; WAC 296-155-548 Design and Installation Requirements; WAC 296-155-549 Personnel Lifting Hoisting Equipment; WAC 296-155-551 Inspections Maintenance and Testing; WAC 296-155-552 Employer Responsibilities AGC: Safety and Health Manual Ch. 15 Cranes and Rigging This form supports compliance with the above regulations. Retain completed forms for minimum 3 years per 29 CFR 1904.33 and WAC 296-27.

Prepared by: ________________________ | Title: ________________________ | Date: ________________________

Reviewed by: ________________________ | Title: ________________________ | Date: ________________________