MH CONSTRUCTION, INC.

Industrial Safety & Health Program (MISH)

FORM-MISH-44 | MISH 44 | Struck-By Caught-In Pre-Task JHA

Rev. 3 | Effective: 04/07/2026 | Safety Officer: Matt Ramsey

| Project Name / Number: |  | Job Site Address: |  |
| --- | --- | --- | --- |
| Date: |  | Supervisor / Foreman: |  |
| Employee Name(s): |  | Subcontractor: |  |

Task Description: ________________________________________________________________________________

| Pre-Task Hazard Identification: ☐ Heavy equipment operation in area ☐ Backing vehicles / blind spots ☐ Overhead loads / crane lifts ☐ Excavation / trenching activities ☐ Material handling / staging ☐ Traffic control / public roads ☐ Pinch points / rotating parts ☐ Unsecured materials / tools ☐ Congested work area ☐ Poor visibility / lighting | Required PPE: ☐ Hard Hat ☐ Safety Glasses ☐ High-Visibility Vest (Class 2/3) ☐ Steel/Composite Toe Boots ☐ Work Gloves ☐ Hearing Protection ☐ Face Shield ☐ Fall Protection |
| --- | --- |

Job Hazard Analysis:

| Step # | Task Step Description | Potential Hazard | Hazard Control Measure | Responsible Party |
| --- | --- | --- | --- | --- |
| 1 | Mobilize equipment to work area | Struck by moving equipment, blind spots | Use spotter when backing, high-vis vests required, establish exclusion zone | Operator / Spotter |
| 2 | Unload materials from delivery truck | Crushed by falling materials, pinch points | Inspect rigging, stay out of fall zone, use tag lines, clear communication | Rigger / Operator |
| 3 | Excavation and trenching | Caught in cave-in, struck by excavator bucket | Shoring/shielding in place, stay out of swing radius, barricade open trenches | Competent Person |
| 4 | Working near active roadway | Struck by public traffic | Set up proper traffic control plan (MUTCD), use concrete barriers, flaggers | Traffic Control Supervisor |
| 5 | Operating hand/power tools | Caught in rotating parts, struck by flying debris | Guards in place, proper PPE (face shield), no loose clothing/jewelry | Worker |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

Emergency Information: Nearest Hospital: ____________________________________ Emergency Number: 911 Site First Aid Location: ________________________________

Regulatory Authority OSHA: 29 CFR 1926 Subpart O; 29 CFR 1926.600 Equipment General; 29 CFR 1926.20; OSH Act General Duty Clause Section 5(a)(1) OSHA Fatal Four WISHA: WAC 296-155-610 Motor Vehicles on Construction Sites; WAC 296-880 Fall Protection overhead hazards; WAC 296-155-035 General Requirements safe place standards AGC: Safety and Health Manual Ch. 27 Struck-By and Caught-In Hazards This form supports compliance with the above regulations. Retain completed forms for minimum 3 years per 29 CFR 1904.33 and WAC 296-27.

| Prepared by: ________________________ | Title: ________________________ | Date: ________________________ |
| --- | --- | --- |
| Reviewed by: ________________________ | Title: ________________________ | Date: ________________________ |